OpenROAD VLSI Design - II

Kevin Umanzor

Talha Ibrahim

Giovani Giagnacovo

Githin Johny

Functional System Requirements

REVISION – 1

04 October 2025

Functional System Requirements

for

OpenROAD VLSI Design II

Prepared by:

Kevin, Talha, Githin, & Giovani                   10/04/25

Author	Date

Approved by:

Kevin Umanzor                    10/04/25

Project Leader	Date

_____________________________

John Lusher, P.E.	Date

Hailey Ward                           10/04/25

T/A      	Date

Change Record

| Rev. | Date | Originator | Approvals | Description |
| --- | --- | --- | --- | --- |
| 0 | 09/30/25 | Kevin Umanzor Talha Ibrahim Giovani Giagnacovo Githin Johny |  | Draft Release & Overview |
| 1 | 10/4/25 | Kevin Umanzor Talha Ibrahim Giovani Giagnacovo Githin Johny |  | Finalized for submission |

Table of Contents

1. Introduction	6

1.1. Purpose and Scope	6

1.2. Responsibility and Change Authority	7

2. Applicable and Reference Documents	8

2.1. Applicable Documents	8

2.2. Reference Documents	8

2.3. Order of Precedence	8

3. Requirements	9

3.1. System Definition	9

3.2. Characteristics	10

3.2.1. Functional / Performance Requirements	10

3.2.2. Physical Characteristics	11

3.2.3. Electrical Characteristics	11-12

3.2.4. Environmental Requirements	12

3.2.5. Failure Propagation	12

4. Support Requirements	13

Appendix A: Acronyms and Abbreviations	14

Appendix B: Definition of Terms	15

List of Tables

No table of figures entries found.

List of Figures

Figure 1.  Your Project Conceptual Image	1

Figure 2.  Block Diagram of System	4

# Introduction

## Purpose and Scope

This specification defines the technical requirements for the development items and support subsystems delivered to the client for the project.  Figure 1 shows a representative integration of the project in the proposed CONOPS. The verification requirements for the project are contained in a separate Verification and Validation Plan.

Figure 1.  System Overview of 16-bit RISC-V

The following definitions differentiate between requirements and other statements.

Shall:  		This is the only verb used for the binding requirements.

Should/May:  	These verbs are used for stating non-mandatory goals.

Will:  		This verb is used for stating facts or declaration of purpose.

## Responsibility and Change Authority

The team leader, Kevin Umanzor, is the one who is responsible for ensuring that the project is built within the constraints that have been given. When considering the authority to who is able to make changes to the project, those are given solely to the team leader along with Professor Kevin Nowka.

# Applicable and Reference Documents

## Applicable Documents

| Document Number | Revision/Release Date | Document Title |
| --- | --- | --- |
| UCB/EECS-2016-1 | 01/03/2016 | Design of the RISC-V Instruction Set Architecture |

## Reference Documents

The following documents are reference documents utilized in the development of this specification.  These documents do not form a part of this specification and are not controlled by their reference herein.

| Document Number | Revision/Release Date | Document Title |
| --- | --- | --- |
| FIND | 2019 | The OpenROAD Project: Unleashing Hardware Innovation |

## Order of Precedence

In the event of a conflict between the text of this specification and an applicable document cited herein, the text of this specification takes precedence without any exceptions.

All specifications, standards, exhibits, drawings or other documents that are invoked as “applicable” in this specification are incorporated as cited.  All documents that are referred to within an applicable report are considered to be for guidance and information only, except ICDs that have their relevant documents considered to be incorporated as cited.

# Requirements

## System Definition

Figure 2.  Block Diagram of System-

The block diagram provides a high-level, generalized view of the main subsystems of our 16-bit RISC-V processor. Instructions flow through four stages: Fetch, where the next instruction is retrieved from memory; Decode, where the instruction is interpreted and operands are read from the register file; Execute, where the ALU carries out arithmetic, logic, or branch operations; and Memory/Write Back, where results are written to memory or updated in the registers. These stages are interconnected in a pipeline, with control and data passed sequentially and feedback ensuring continuous program execution. This modular organization allows the processor to efficiently execute RISC-V instructions, providing a simple yet extensible foundation for building and testing a functional 16-bit CPU.

Beyond implementing the logic, building this 16-bit CPU also establishes the foundation needed to move into physical design. With a functioning core, we can use the OpenROAD flow to generate the complete GDSII layout, bridging the gap between abstract processor design and actual chip implementation. This process ensures that our project is not just about verifying instruction execution in simulation, but also about creating a working silicon-ready layout that integrates all stages of the design cycle, evaluating the use and efficiency of the OpenROAD tools along the way.

## Characteristics

### Functional / Performance Requirements

#### Instruction Set architecture (ISA) Support

This processor should abide by the core standards, conventions, and requirements of the RISC-V instruction set architecture. The processor will support instructions derived from the I32E embedded version of the RISC-V architecture.

Rationale:  The choice to use the RISC-V ISA was made because it is an open, modular, and well-documented standard that provides flexibility for customization while ensuring compatibility with a widely adopted architecture.

#### Clock Frequency

The RISC-V processor should operate at a clock frequency of 10 MHz.

Rationale:  The choice of a 10 MHz clock frequency was made to balance performance with ultra-low-power operation, ensuring the processor can execute instructions reliably without exceeding the strict power budget.

#### Datapath Width

The RISC-V processor that will be implemented will have the capability to support 16-bit instructions. These instructions will be based on the official RISC-V instruction set for an embedded processor.

Rationale:  The choice of having 16-bit instructions was made because of the restriction of the 2-tile maximum layout and also considering the timeline for the project.

#### Register File

The RISC-V processor shall be able to support a system register file consisting of 16 general-purpose registers.

Rationale:  Given the 16-bit instruction constraint, only 16 registers can be referenced directly in the opcode and instruction encoding, allowing for efficient use of all available bits per instruction while maintaining simplicity in the design.

#### Power Requirements

The RISC-V processor shall not exceed 200μW during active power consumption, remaining under this threshold during idle periods.

Rationale:  The energy per instruction is expected to remain below 20 pJ/instruction, making the design suitable for lightweight, embedded, or educational applications where efficiency is critical and performance is not the highest priority.

### Physical Characteristics

#### Tile-size constraint

The OpenROAD VLSI microprocessor shall occupy a maximum of 2 tiles on the Tiny Tapeout wafer provided by ChipFoundry.io. This maximum area is based on the simplicity of the processor design and lack of many additional components.

Rationale: This maximum area is defined by what should be necessary for a typical 16-bit RISC microprocessor design. This ensures that the chip does not occupy an unnecessarily large space.

#### Volume Envelope

The volume of the OpenROAD VLSI chip is expected to fall between 0.005 mm^3 - 0.04 mm^3.

Rationale: This maximum volume is defined by what should be necessary for a typical 16-bit RISC microprocessor design. This ensures that the chip does not occupy an unnecessarily large space.

### Electrical Characteristics

##### Power Consumption

- The maximum power of the system shall not exceed 200μW.

Rationale:  This is a requirement specified by our customer due to constraints of their system in which the Search and Rescue System is integrating.

##### Input Voltage Level

The input voltage for the RSIC-V 16 bit shall be +3.3V to +5.0V

Rationale:  These requirements are specified by the skywater technology and their constraints for designing 130nm chips.

#### Outputs

##### I/O Pins

The OpenROAD VLSI microprocessor will include I/O pins for the I2C (inter-Integrated circuit) to connect to external I/O devices at a specified clock line. Additionally pins will be added for the SPI to ensure bytes are being properly sent/received by the external I/O device connected. A USB interface will also be provided for power and data transfer between other devices.

Rationale:  These I/O pins are included to support the USB interface, I2C, and SPI interface requirements set for the microprocessor.

### Environmental Requirements

The RISC-V 16-bit processor shall have Environmental Requirements specified in the following sections.

#### Thermal

The 16-bit fabricated chip shall operate in a temperature range from -55℃ to + 125℃.

Rationale:  This is a requirement specified by the company that will fabricate the chip, Skywater Technologies.

### Failure Propagation

The RISC-V 16-bit processor shall have failure propagations specified in the following sections.

      - Reset for Fault Recovery

The RISC-V 16-bit processor shall include a reset pin that allows the processor to recover from any unexpected states.

Rationale: The reset pin ensures the processor can be reliably returned to a known state after unexpected errors, improving system stability and ease of recovery.

      - ESD Protection

The RISC-V 16-bit processor shall include electrostatic discharge (ESD) protection diodes on all I/O pads to prevent internal damage caused by static discharge events.

Rationale: The ESD protection ensures that the internal logic of the processor remains unaffected by transient high-voltage events, preventing latch-up or permanent device failure and improving overall chip reliability.

# Support Requirements

An overview of the implementation of the 16-bit RISC-V architecture shall be provided, including the design details, which was built keeping in mind the 16-bit size, Verilog code & testbenches for each major component, and the instructions for simulating the code onto OpenROAD/OpenLane.

# Appendix A: Acronyms and Abbreviations

ALU		Arithmetic Logical Unit

CPU		Central Processing Unit

GDSII		Graphic Data System II

ISA			Instruction Set Architecture

I2C			Inter-Integrated Circuit

RISC-V		Reduced Instruction Set Computing V

SPI			Serial Peripheral Interface

μW			microWatts

VLSI		Very Large Scale Integration

V			Volt

# Appendix B: Definition of Terms

Architecture		Architecture, as defined and referenced in this document, refers to the

functional and organization aspects of a computer system