OpenROAD VLSI Design - II

Kevin Umanzor

Talha Ibrahim

Giovani Giagnacovo

Githin Johny

Interface Control Document

REVISION – Draft

25 January 2018

Interface Control Document

for

OpenROAD VLSI Design Team 2

Prepared by:

Kevin, Talha, Githin, & Giovani         10/01/2025

Author	             Date

Approved by:

Kevin Umanzor                10/04/2025

Project Leader	Date

_____________________________

John Lusher II, P.E.	Date

Hailey Ward                             10/04/2025

T/A      	Date

Change Record

| Rev. | Date | Originator | Approvals | Description |
| --- | --- | --- | --- | --- |
| 0 | 10/1/25 | Kevin Umanzor Talha Ibrahim Giovani Giagnacovo Githin Johny |  | Draft Release & Overview |
| 1 | 10/4/25 | Kevin Umanzor Talha Ibrahim Giovani Giagnacovo Githin Johny |  | Finalized for submission |

Table of Contents

No table of figures entries found.	4

1. Overview	6

2. References and Definitions	7

2.1. .References	7

2.2. Definitions	7

3. Physical Interface	8

3.1. Tile-Size Constraint	8

3.1.1. Dimension of subsystems	8

3.2. Mounting Locations	8

4. Thermal Interface	9

4.1. Operating Temperature Range	9

5. Electrical Interface	10

5.1. Primary Input Power	10

5.2. Signal Interfaces	10

5.3. Video Interfaces	10

5.4. User Control Interface	10

6. Communications / Device Interface Protocols	11

6.1. Fetch	11

6.2. Decode	11

6.3. Execute	11

6.4. Memory/Write Back	11

List of Tables

No table of figures entries found.

List of Figures

# No table of figures entries found.

# Overview

This specification covers the interfaces and interactions between the high-level subsystems of the 16-bit RISC-V microprocessor. This document will give specific instructions for module-to-module communication between the following subsystems: fetch, decode, execution, and memory/write back. This document serves the purpose of streamlining subsystem integration and reducing potential risks that may arise due to poorly defined module communication boundaries.

# References and Definitions

## References

UCB/EECS-2016-1

Design of the RISC-V Instruction Set Architecture

01/03/2016

Change Notice 2

03 Jan 2016

American National Standard for VME64 (ANSI/VITA 1-1994 (R2002))

4 Apr 1995

American National Standard for VME64 Extensions (ANSI/VITA 1.1-1997)

7 Oct 1998

## Definitions

ALU			Arithmetic Logical Unit

RTL				Register-Transfer Level

CPU			Central Processing Unit

GDSII			Graphic Data System II

ISA				Instruction Set Architecture

I2C				Inter-Integrated Circuit

RISC-V			Reduced Instruction Set Computing V

SPI				Serial Peripheral Interface

μW				microWatts

VLSI			Very Large Scale Integration

V				Volt

# Physical Interface

## Tile-Size Constraint

The RISC-V microprocessor shall occupy a maximum of 2 tiles on the Tiny Tapeout water from ChipFoundry.io. This constraint shall ensure that the design is able to fit within the 2 tile fabrication size while also supporting 16-bit instructions.

### Dimension of subsystems

Each subsystem shall be described and verified at an RTL level in Verilog and therefore shall not have individual physical dimensions. The physical dimensions will only be applicable to the final fabricated chip, which has a constraint of being 2-tile TinyTapeout area.

## Mounting Locations

Mounting locations are not applicable to this project. The scope of this project is to design a RISC-V processor and have it ready for fabrication. Mounting would be something that is later determined when considering PCB integrations, which is out of the scope for this.

# Thermal Interface

The system itself shall not require any additional thermal interfaces, like cooling, air circulation, or heatsinks. Since the power that will be dissipated will be in the mW, there will be no need to include an external thermal cooling system.

  - Operating Temperature Range

The 16-bit RISC-V processor is expected to operate within a temperature range of −55 °C to +125 °C, consistent with the SkyWater 130 nm CMOS process specifications. This range defines the limits within which the chip can maintain reliable performance without degradation, though operation closer to nominal room temperature is recommended for optimal stability and safety.

# Electrical Interface

## Primary Input Power

The primary input power for the openROAD VLSI chip is expected to reach a maximum of about 200μW. This amount of power is enough to ensure functionality of a 2-tile skywater chip.

## Signal Interfaces

The openROAD VLSI chip has numerous signal interfaces including a UART console, SPI master, I2C master, external I/O interface, and a USB interface. These interfaces define timing constraints and provide a means for the chip to move data and control between blocks.

## User Control Interface

The openROAD VLSI chip provides reset and debug pins for Users to directly control the chip externally. The chip also features an external clock for users to provide clock inputs to the system.

## Clock Interface

The openROAD VLSI chip will require an external clock with an estimated target frequency of about 10MHz. The clock interface is expected to be able to support a range of 1-20MHz to ensure a reliable frequency for chip functionality.

# Communications / Device Interface Protocols

## Fetch

The Fetch stage retrieves the next instruction from instruction memory using the current PC. The main output of this stage is the 16-bit instruction word instr[15:0], which represents the instruction to be executed.

## Decode

The Decode stage processes the 16-bit instruction by dividing it into distinct fields that drive control and register selection. The instruction fields consist of the opcode (instr[15:13], three bits defining the instruction type), the func field (instr[12:9], four bits specifying sub-operations), and register identifiers: rs1 (instr[8:6]), rs2 (instr[5:3]), and rd (instr[2:0]). The opcode and func values are forwarded to the control unit, which generates signals such as ALU control, register write enable, and memory access commands. The register identifiers are sent to the register file to select source and destination registers to perform operations on.

## Execute

The Execute stage performs the arithmetic and logical operations defined by the decoded instruction. Its primary inputs are rs1_data[15:0] from the register file, rs2_data[15:0] or an immediate constant imm[15:0], and the ALU control signal alu_ctrl[3:0] from the control unit. Based on these inputs, the ALU produces the result alu_result[15:0] and may generate condition flags such as Zero or Negative, which are used for branch decisions.

## Memory/Write Back

The Memory and Write-Back stage handles data transfers to and from memory and finalizes the result of each instruction. For memory operations, the ALU result alu_result[15:0] provides the memory address, while rs2_data[15:0] supplies data to be written on store instructions. Control signals such as MemRead and MemWrite determine whether a read or write occurs. The output from memory, mem_data_out[15:0], or alternatively the ALU result, is selected through a write-back multiplexer. The chosen value is then returned to the register file as rd_data[15:0], marking the completion of the instruction’s execution cycle.